PROJECT PROPOSAL & SYSTEM DESIGN SPECIFICATION

AI-POWERED COMMUNITY ENERGY
MANAGEMENT & GRID ORCHESTRATOR

A Digital-Twin Simulation of Carbon-Aware Scheduling, Peak Shaving,
Distributed Energy Resources, Battery Dispatch, and Agentic AI

| Document Attribute | Specification |
| --- | --- |
| Document Type | 100-Mark Project Proposal / Technical Design Specification |
| Project Class | AI + Energy Systems + Optimization + Digital Twin |
| Primary Demonstration | Residential community / apartment society with DERs and a flexible data-center load |
| Simulation Resolution | 15-minute intervals (96 timesteps per simulated day) |
| Physical Grid | Simulated; no physical power electronics required |
| Decision Layer | Industry-aligned optimization and constraint engine |
| AI Layer | Forecasting ML + tool-using Agentic AI + LLM natural-language interface |
| Target Build Window | 1–2 days for a focused proof-of-concept |
| Implementation Philosophy | Realistic software architecture; synthetic physical/workload data |
| Status of Numerical Parameters | Tentative demonstration values; configurable, not field-calibrated |

Prepared as an implementation-ready blueprint for an AI-assisted software build

# 1. Executive Summary

This proposal defines an AI-driven Community Energy Management and Grid Orchestrator implemented as a software digital twin. The system models a small electricity community containing residential apartments, flexible electric-vehicle charging, a data-center workload, commercial/common-area loads, rooftop solar, battery energy storage, and an external utility grid.

The project deliberately separates four responsibilities. First, a simulation engine creates the virtual physical/operational environment. Second, forecasting models estimate future load and renewable availability. Third, a deterministic optimization engine schedules flexible loads and battery actions under hard operational constraints. Fourth, an Agentic AI layer uses an LLM as a natural-language orchestrator that retrieves system state, calls analytical tools, invokes the optimizer, runs what-if simulations, and explains the resulting decision to a human.

The project addresses two related but distinct objectives: peak shaving and carbon-aware scheduling. Peak shaving reduces the maximum demand imposed on the grid or community connection. Carbon-aware scheduling moves flexible consumption toward periods with lower modeled grid carbon intensity, typically when the generation mix contains a larger proportion of low-carbon generation. These objectives are combined with energy cost, renewable utilization, battery constraints, user deadlines, and comfort/priority constraints.

Core design principle The LLM does not directly calculate or invent the electrical schedule. The LLM interprets intent and orchestrates tools; the optimization solver produces the constraint-safe schedule; the simulator evaluates its consequences.

The proposed system is intentionally not a full electromagnetic power-flow simulator. It is an Energy Management System / DER orchestration simulation in which the physical world is abstracted into power, energy, availability, state-of-charge, carbon-intensity, price, and workload variables. This makes the project achievable within one to two days while preserving the architecture and decision logic needed for a credible industry-oriented demonstration.

# 2. Problem Statement

## 2.1 Background

Electricity demand is not constant. Residential demand typically changes with occupancy and daily routines; electric-vehicle charging can create concentrated peaks; commercial and common-area loads have their own schedules; and data-center workloads may contain deferrable computation. At the same time, solar and other renewable generation are variable. The external grid can therefore experience periods of high demand, constrained capacity, changing electricity prices, and changing carbon intensity.

## 2.2 Practical problem

A community may have several flexible loads but no coordinated mechanism to determine when those loads should operate. A naive policy can charge all EVs at the same time, run discretionary computation during a system peak, or miss periods of high solar generation. Conversely, blindly shifting every load to the lowest-carbon period can violate deadlines, create a new peak, or reduce resident comfort.

## 2.3 Formal problem statement

Design and implement a digital energy-management system that forecasts community demand and renewable availability, computes time-varying carbon intensity and energy cost, schedules flexible loads and battery actions under explicit constraints, evaluates the schedule in a virtual grid, and exposes the system through an Agentic AI interface capable of answering questions and running controlled what-if scenarios.

# 3. Objectives and Success Criteria

| Objective | Implementation requirement | Success criterion |
| --- | --- | --- |
| Peak shaving | Reduce maximum grid import through flexible scheduling and battery dispatch. | Optimized peak lower than baseline for test scenarios without violating hard constraints. |
| Carbon-aware scheduling | Use time-varying modeled grid carbon intensity in the objective. | Lower associated operational CO₂e than baseline for carbon-focused scenarios. |
| Renewable utilization | Prefer local renewable consumption/storage when economically and operationally appropriate. | Higher renewable self-consumption or reduced curtailment in selected scenarios. |
| Cost awareness | Include time-varying energy price as an objective term. | Cost reduction in cost-focused scenarios. |
| User constraints | Respect deadlines, availability windows, required energy and priorities. | Zero hard-constraint violations in feasible scenarios. |
| Battery management | Respect SOC, power, efficiency and reserve constraints. | SOC remains within bounds; no impossible charging/discharging. |
| AI forecasting | Predict short-horizon demand and solar availability. | Forecasting pipeline executes and produces inputs to optimization. |
| Agentic AI | LLM uses tools rather than hallucinating system state. | Agent can inspect state, request optimization, run what-if simulation, explain results. |
| Explainability | Every schedule change has machine-readable reasons. | User can see objective contribution and constraints behind recommendations. |
| Reproducibility | Seeded simulator and configuration-driven scenarios. | Same seed/configuration reproduces the same run. |

# 4. Scope and Non-Scope

## 4.1 In scope

24-hour and multi-day discrete-time energy simulation at 15-minute resolution.

Apartment/community base demand profiles.

Flexible EV charging, washing-machine-like loads, water pumping, HVAC flexibility, and data-center batch workloads.

Rooftop solar generation profile and optional wind profile as a later extension.

Battery energy storage with SOC and charge/discharge constraints.

Utility-grid import/export abstraction and grid capacity constraint.

Time-varying electricity price and modeled grid carbon intensity.

Baseline rule-based scheduling and optimized scheduling.

Constraint-based optimization using OR-Tools CP-SAT or a suitable linear/MIP solver.

ML load/renewable forecasting using a lightweight tabular model.

LLM-based Agentic AI with tool/function calling.

What-if scenario engine and comparative metrics.

Interactive dashboard showing grid state, forecasts, schedule, battery, carbon, cost and AI explanations.

## 4.2 Explicitly out of scope for the 1–2 day build

Full AC/DC power-flow simulation, transient stability, protection coordination, harmonic analysis, or electromagnetic simulation.

Direct control of physical breakers, inverters, meters, EV chargers or building management systems.

Safety-certified autonomous control.

Real utility SCADA/ADMS integration.

Live utility tariffs or live carbon-intensity feeds as mandatory dependencies.

Training a large neural network from scratch.

A production-grade multi-tenant cloud deployment.

Important boundary The system is a software simulation and decision-support prototype. It should not be represented as a certified grid-control system or as a replacement for utility operational technology.

# 5. Domain Model: What the Virtual Community Represents

The virtual community is designed to be understandable from an apartment-society perspective while still representing a modern distributed-energy environment. The community is connected to the utility grid at a single point of common coupling (PCC). Local solar and battery resources sit behind the community connection.

UTILITY / GRID

│

PCC / GRID CONNECTION

│

┌──────────────┴──────────────┐

│       COMMUNITY BUS         │

│                             │

┌───┴────┐                    ┌───┴────┐

│  Solar │                    │Battery │

└───┬────┘                    └───┬────┘

│                              │

┌──────┼───────────────┬──────────────┼──────┐

│      │               │              │      │

Apartments   EV Chargers  Common Area  Data Center  Commercial

│              │            │             │          │

Fixed +      Flexible      Pumps/Lifts    Flexible    HVAC/etc.

Flexible       Load          /Lighting     Workloads

## 5.1 Asset classes

| Asset / Load | Type | Flexible? | Core attributes |
| --- | --- | --- | --- |
| Apartment base load | Demand | No / limited | kW profile, occupancy pattern |
| EV charger | Demand | Yes | arrival, departure, energy required, max kW, priority |
| Water pump | Demand | Yes | required energy, operating window, max kW |
| HVAC | Demand | Partially | comfort band, baseline kW, flexibility factor |
| Washing machine | Demand | Yes | duration, energy, deadline |
| Data-center batch job | Demand | Yes | work units, energy/work unit, deadline, interruptibility, priority |
| Critical data-center workload | Demand | No | must-run / immediate |
| Rooftop solar | Generation | Weather-dependent | forecast kW, actual kW, capacity |
| Battery | Storage | Yes | capacity kWh, SOC, charge/discharge kW, efficiency, reserve |
| Utility grid | Supply/import/export | System boundary | capacity, price, carbon intensity |

# 6. Carbon Model: Apartment-Level Explanation and System Formulation

## 6.1 What does “carbon” mean here?

Electricity consumed by an apartment does not itself contain CO₂. The associated operational emissions are attributed to the generation mix supplying the electricity. Fossil-fuel generation releases CO₂ when carbon-containing fuels are combusted. Solar and wind generation have no direct combustion emissions during operation. The simulation therefore assigns a time-varying grid carbon-intensity value to imported electricity.

## 6.2 Why carbon intensity changes with time

The grid is an interconnected system in which many generation resources may operate at the same time. The community does not receive a physically separable “solar wire” or “coal wire”. Instead, the system models the grid as an aggregate generation mix. At a solar-rich period, the modeled carbon intensity can be lower; when solar output falls and fossil generation supplies a larger share of marginal/aggregate demand, the modeled intensity can be higher.

## 6.3 Operational carbon calculation

For each timestep t:

GridImportEnergy_t = GridImportPower_t × Δt

OperationalCO2e_t = GridImportEnergy_t × CarbonIntensity_t

TotalOperationalCO2e = Σ_t OperationalCO2e_t

For the demonstration, carbon intensity is expressed in gCO₂e/kWh. The project should clearly label it as a modeled/synthetic grid carbon-intensity signal unless a validated external dataset is used.

## 6.4 Illustrative apartment example

| Period | Apartment consumption | Modeled carbon intensity | Associated operational emissions |
| --- | --- | --- | --- |
| Solar-rich afternoon | 10 kWh | 200 gCO₂e/kWh | 2.0 kgCO₂e |
| High-demand evening | 10 kWh | 600 gCO₂e/kWh | 6.0 kgCO₂e |

The example does not claim that these exact values apply to a particular real utility. They are intentionally illustrative and configurable. The important relationship is that the same amount of flexible energy can have a different modeled emissions consequence depending on when it is imported.

# 7. Tentative Load Model and Engineering Assumptions

The following values are demonstration assumptions for a medium-sized apartment community. They are not electrical design specifications. The simulator must load them from configuration so the reviewer can change the community size without rewriting code.

## 7.1 Community configuration

| Parameter | Tentative value | Purpose |
| --- | --- | --- |
| Apartments | 100 | Residential population scale |
| Average apartment baseline demand | 0.8–1.2 kW average; configurable profile | Synthetic base demand |
| Residential evening peak contribution | Approx. 1.5–2.5 kW/apartment at peak | Creates realistic daily peak |
| EV chargers | 40 | Flexible demand |
| EV charger rated power | 7.2 kW | Single-phase/typical illustrative AC charging abstraction |
| EV daily energy request | 8–24 kWh/session | Synthetic arrival/departure requirement |
| Community solar | 250 kW DC/AC modeled output cap | Local renewable generation |
| Battery | 500 kWh usable energy | Peak shaving / solar shifting |
| Battery power | 250 kW charge/discharge | Configurable power limit |
| Battery round-trip efficiency | 90% | Illustrative storage loss |
| Community/data-center flexible load | 100–300 kW | Deferrable computation |
| Common-area/commercial peak | 100–250 kW | Pumps, HVAC, lighting, shops etc. |
| PCC import limit | 2.5 MW | Creates a meaningful grid constraint |
| Simulation interval | 15 minutes | 96 intervals/day |

## 7.2 Basic power/energy relationship

Energy (kWh) = Power (kW) × Time (hours)

For a 15-minute interval:

Δt = 15 / 60 = 0.25 h

Example:

A 7.2 kW EV charger running for 15 minutes consumes:

7.2 × 0.25 = 1.8 kWh

## 7.3 Residential baseline construction

Each apartment receives a synthetic daily profile generated from a base curve plus bounded variability. The profile should have lower overnight demand, a morning increase, a daytime plateau/decline, and an evening peak. Random noise must be seeded for reproducibility.

ApartmentBaseLoad_t =

OccupancyProfile_t

+ ApplianceProfile_t

+ HVACProfile_t

+ RandomNoise_t

CommunityResidentialLoad_t =

Σ apartment_load_i,t

## 7.4 Flexible-load energy calculation examples

| Workload | Assumption | Energy calculation |
| --- | --- | --- |
| EV session | 7.2 kW, 3 h effective operation | 21.6 kWh |
| Washing machine | 1.2 kW average, 1 h | 1.2 kWh |
| Water pump | 50 kW, 1 h equivalent | 50 kWh |
| Data-center batch | 200 kW equivalent, 2 h | 400 kWh |
| Community HVAC flexibility | Baseline 180 kW, 20% shiftable | 36 kW flexible component |

# 8. Renewable Generation and Grid Model

## 8.1 Solar generation

Solar output is represented as a normalized daylight curve multiplied by installed capacity and a weather/cloud factor. The model should peak around midday and fall to zero at night.

SolarPower_t =

SolarCapacity × SolarShape(hour_t) × WeatherFactor_t

0 ≤ SolarPower_t ≤ SolarCapacity

## 8.2 Optional wind

Wind is optional for the first build. If implemented, use a synthetic stochastic profile rather than attempting to model turbine aerodynamics.

## 8.3 Grid import/export balance

CommunityDemand_t

= Residential_t

+ EV_t

+ CommonArea_t

+ Commercial_t

+ DataCenter_t

NetGridNeed_t

= CommunityDemand_t

- SolarUsedDirectly_t

- BatteryDischarge_t

+ BatteryCharge_t

GridImport_t = max(NetGridNeed_t, 0)

GridExport_t = max(-NetGridNeed_t, 0)

For the first implementation, export may be disabled to simplify the objective. A later configuration can permit export with a separate export tariff.

# 9. Battery Energy Storage Model

The battery provides a controllable flexibility resource. It can absorb surplus solar or low-cost/low-carbon energy and discharge during peak periods. The simulator must prevent physically impossible simultaneous charge and discharge.

SOC_{t+1} =

SOC_t

+ η_charge × ChargePower_t × Δt

- (DischargePower_t × Δt / η_discharge)

Constraints:

SOC_min ≤ SOC_t ≤ SOC_max

0 ≤ ChargePower_t ≤ P_charge_max

0 ≤ DischargePower_t ≤ P_discharge_max

ChargePower_t × DischargePower_t = 0  (implemented via binary logic)

SOC_final ≥ reserve target (optional)

For a 500 kWh battery with a 20% minimum SOC and 90% round-trip efficiency, the system should reserve a configurable energy margin rather than attempting to use 100% of nominal capacity.

# 10. Core Optimization Formulation

The central scheduling problem is a constrained multi-objective optimization problem. The proposed implementation uses Google OR-Tools CP-SAT where binary/integer scheduling decisions dominate, or an MIP/LP formulation where continuous energy variables are more natural. OR-Tools explicitly supports scheduling, linear/mixed-integer programming and constraint optimization.

## 10.1 Decision variables

| Variable | Meaning | Type |
| --- | --- | --- |
| x_load,t | Whether a discrete flexible load runs in timestep t | Binary |
| p_load,t | Power assigned to a flexible load | Continuous / discretized |
| charge_t | Battery charging power | Continuous / discretized |
| discharge_t | Battery discharging power | Continuous / discretized |
| soc_t | Battery state of charge | Continuous / discretized |
| grid_import_t | Grid import power | Continuous / discretized |
| grid_export_t | Grid export power | Continuous / discretized |
| peak | Maximum grid import over horizon | Continuous / integer |

## 10.2 Multi-objective cost function

The optimizer should minimize a weighted normalized objective so that cost, carbon and peak penalties are comparable.

Minimize:

J =

w_cost   × NormalizedEnergyCost

+ w_carbon × NormalizedCO2

+ w_peak   × NormalizedPeakImport

+ w_grid   × NormalizedGridStress

+ w_discomfort × NormalizedUserDisutility

+ w_curtail × NormalizedRenewableCurtailment

where:

w_cost + w_carbon + w_peak + w_grid + w_discomfort + w_curtail = 1

Three default policy profiles should be exposed: Balanced, Carbon Priority, and Peak Protection. These are not separate algorithms; they are different objective-weight configurations.

| Policy | Cost weight | Carbon weight | Peak weight | Comfort/disutility | Purpose |
| --- | --- | --- | --- | --- | --- |
| Balanced | 0.25 | 0.25 | 0.25 | 0.20 | General community operation |
| Carbon Priority | 0.10 | 0.50 | 0.20 | 0.20 | Minimize modeled operational CO₂ |
| Peak Protection | 0.15 | 0.15 | 0.50 | 0.20 | Protect PCC/grid capacity |

## 10.3 Hard constraints

Every flexible workload must receive its required energy before its deadline.

A load can only operate inside its allowed availability window.

Critical loads must run according to their fixed profile.

Grid import cannot exceed PCC capacity.

Battery SOC cannot exceed maximum or fall below minimum.

Battery charge/discharge power cannot exceed ratings.

Battery cannot charge and discharge simultaneously.

EV energy requirement must be satisfied before departure.

Data-center workload must satisfy its completion deadline.

User comfort constraints must be respected for semi-flexible HVAC.

## 10.4 Why a real optimizer is preferable to an LLM

A language model is probabilistic and is not the correct authority for numerical constraint satisfaction. The optimizer can prove feasibility or report infeasibility and can expose solver status. OR-Tools CP-SAT, for example, returns explicit statuses such as OPTIMAL, FEASIBLE, INFEASIBLE and UNKNOWN.

# 11. Baseline System for Scientific Comparison

The project must not compare the optimized schedule against an undefined notion of 'normal'. Implement a deterministic baseline scheduler first.

Baseline rules:

1. Apartment base loads run normally.

2. EVs begin charging immediately upon arrival.

3. Washing machines run at a default user-selected time.

4. Data-center batch jobs execute at fixed preferred hours.

5. Battery follows a simple rule: charge from excess solar; discharge at fixed evening peak.

6. No carbon-aware optimization.

7. No global coordination across flexible loads.

The same simulation seed, load profiles and weather conditions must then be replayed under the optimized controller. This creates a controlled A/B comparison.

# 12. Forecasting ML Layer

## 12.1 Purpose

Forecasting is used to make the optimizer proactive rather than reactive. The initial system should predict the next 24 hours of aggregate load and solar output using a lightweight tabular ML model.

## 12.2 Features

| Feature group | Example features |
| --- | --- |
| Time | hour, minute bucket, day-of-week, weekend flag |
| Lagged load | load t-1, t-4, t-24h, rolling mean |
| Weather proxy | temperature, cloud factor, solar availability factor |
| Occupancy proxy | weekday/weekend, morning/evening indicator |
| Solar history | previous solar output, rolling solar mean |

## 12.3 Model

Recommended first implementation: XGBoost or LightGBM regression for aggregate load and a second model for solar output. If dependency installation becomes a problem, use Random Forest or a simple gradient boosting model. The project should prioritize the end-to-end system over squeezing out marginal forecast accuracy.

## 12.4 Forecast workflow

Historical / synthetic observations

↓

Feature engineering

↓

Train / validation split

↓

Forecast next 96 intervals

↓

Optimizer consumes forecast

↓

Schedule

↓

Simulator reveals actual outcome

↓

Forecast error + operational metrics

# 13. Agentic AI Architecture

Agentic AI is included, but its role is deliberately bounded. The agent is a tool-using orchestration layer. It can interpret a human request, inspect the current virtual grid, call forecasting and carbon tools, invoke the deterministic optimizer, run what-if simulations, compare outcomes, and explain the result.

## 13.1 Agent responsibilities

Interpret natural-language intent.

Identify which system information is required.

Call read-only tools for current grid state, forecasts, battery state and workload state.

Select an optimization policy or adjust approved objective weights within configured bounds.

Call the optimization tool.

Call the scenario simulator for what-if questions.

Compare baseline and optimized results.

Generate a human-readable explanation with explicit numerical evidence.

Never invent live system state; every state claim must come from a tool result.

## 13.2 Agent tool set

| Tool | Input | Output | Mutability |
| --- | --- | --- | --- |
| get_grid_state | timestamp / scenario | load, import, solar, battery, carbon, price | Read-only |
| get_forecast | horizon, variable | 96-step load/solar forecast | Read-only |
| get_flexible_loads | scenario | EV/workload constraints | Read-only |
| get_carbon_profile | horizon | carbon intensity profile | Read-only |
| optimize_schedule | policy + constraints | feasible schedule + metrics | Computation |
| run_simulation | scenario + schedule | time-series outcomes | Computation |
| compare_scenarios | baseline + candidate | delta metrics | Computation |
| explain_schedule | schedule + objective breakdown | structured explanation | Read-only |

## 13.3 Agent decision loop

USER REQUEST

↓

LLM / AGENT

↓

Intent classification

↓

Need current state? ── yes ──→ get_grid_state

↓

Need forecast? ─────── yes ──→ get_forecast

↓

Need carbon? ────────── yes ──→ get_carbon_profile

↓

Need a new schedule? ── yes ──→ optimize_schedule

↓

Need "what if"? ─────── yes ──→ run_simulation

↓

compare_scenarios / explain_schedule

↓

Human-readable response + machine-readable action summary

## 13.4 Example agent interaction

User:

"Tomorrow is forecast to be cloudy. How should we change the community schedule?"

Agent:

1. Retrieve tomorrow's solar forecast.

2. Retrieve tomorrow's aggregate load forecast.

3. Retrieve battery SOC and flexible-load constraints.

4. Generate optimized schedule under Balanced policy.

5. Compare against current baseline.

6. Report expected changes in peak, cost, CO2e and renewable utilization.

Agent response:

"Solar output is forecast to be 32% below the baseline scenario. The optimizer therefore

moves selected EV charging into the remaining solar-rich period, preserves battery reserve,

and delays non-critical data-center work away from the projected evening peak. All EV

departure deadlines remain satisfied."

# 14. OpenAI API Integration Design

The OpenAI API is used for the agent's language reasoning and tool selection. The application server remains the authority for numerical state, optimization, simulation and persistence.

Frontend

↓ HTTPS

FastAPI / Agent Service

↓

OpenAI API (LLM)

↓ tool calls

Internal tools

├── Grid state service

├── Forecast service

├── Carbon service

├── Optimization service

└── Simulation service

The LLM receives tool schemas rather than raw internal implementation details. Tool outputs should be concise, typed, and machine-readable. The backend validates every tool argument before execution. The agent must never be given direct database write authority.

## 14.1 Guardrails

Tool allowlist: the model can call only explicitly registered tools.

Read/write separation: scheduling computation produces a candidate schedule; persistence requires backend validation.

Hard constraints cannot be disabled by natural language.

Objective weights are bounded by server-side configuration.

Every schedule carries a solver status and validation result.

Every what-if scenario uses an isolated scenario ID.

The UI labels synthetic values and simulation results clearly.

API keys are server-side only and never shipped to the browser.

# 15. Complete Software Architecture

┌────────────────────────────────────────────────────────────────────┐

│                         NEXT.JS WEB UI                             │

│ Dashboard │ Scenarios │ Schedules │ Assets │ AI Copilot │ Metrics │

└───────────────────────────────┬────────────────────────────────────┘

│ REST / JSON

▼

┌────────────────────────────────────────────────────────────────────┐

│                         FASTAPI BACKEND                            │

│                                                                    │

│  API Router                                                       │

│      │                                                             │

│      ├── Simulation Service                                       │

│      ├── Forecast Service                                         │

│      ├── Carbon / Tariff Service                                  │

│      ├── Optimization Service                                    │

│      ├── Metrics / Evaluation Service                             │

│      └── Agent Service ──────── OpenAI API                        │

│                                                                    │

└───────────────┬───────────────────────────────┬────────────────────┘

│                               │

▼                               ▼

SQLite / JSON config              OR-Tools Solver

│                               │

└───────────────┬───────────────┘

▼

Simulation Engine

│

▼

Time-series result store

## 15.1 Module boundaries

| Module | Responsibility | Must not do |
| --- | --- | --- |
| simulation/ | Generate and execute virtual world state. | Make optimization decisions. |
| forecasting/ | Train/load models and produce forecasts. | Override hard constraints. |
| optimization/ | Build and solve mathematical scheduling model. | Generate natural-language explanations. |
| carbon/ | Produce time-varying carbon signal and accounting. | Claim real-world emissions without source data. |
| agent/ | LLM orchestration and tool calling. | Directly mutate physical/system state. |
| api/ | Expose validated endpoints. | Contain solver logic. |
| ui/ | Visualization and human interaction. | Perform authoritative calculations. |
| evaluation/ | Compare baseline vs optimized runs. | Alter schedules. |

# 16. Data Model / Database Schema

| Entity | Important fields |
| --- | --- |
| SimulationRun | id, seed, start_time, horizon_steps, configuration_id, status |
| GridSnapshot | run_id, step, timestamp, demand_kw, solar_kw, grid_import_kw, grid_export_kw, carbon_g_per_kwh, price |
| Asset | id, type, name, rated_power_kw, capacity_kwh, flexibility_class |
| FlexibleLoad | id, asset_id, energy_required_kwh, earliest_start, latest_end, max_power_kw, priority, interruptible |
| BatteryState | run_id, step, soc_kwh, charge_kw, discharge_kw |
| Forecast | run_id, step, variable, predicted_value, actual_value, model_version |
| ScheduleAction | run_id, load_id, step, planned_power_kw, reason_code |
| OptimizationRun | id, run_id, policy, weights, solver_status, objective_value, solve_time |
| MetricSnapshot | run_id, total_energy_kwh, peak_kw, cost, co2e_kg, renewable_utilization_pct, violations |
| Scenario | id, parent_run_id, changes_json, created_at |
| AgentSession | id, conversation_id, scenario_id, tool_calls, final_response |

# 17. Backend API Contract

| Method | Endpoint | Purpose |
| --- | --- | --- |
| GET | /api/grid/state | Current virtual grid state |
| GET | /api/grid/timeseries | Historical/current simulated time series |
| GET | /api/forecast?run_id=&horizon= | Load/solar forecast |
| GET | /api/assets | Community assets |
| GET | /api/flexible-loads | Flexible workload constraints |
| POST | /api/simulations | Create a simulation run |
| POST | /api/optimize | Run optimizer with selected policy |
| POST | /api/simulations/what-if | Clone scenario with parameter changes |
| GET | /api/results/{run_id} | Metrics and time series |
| POST | /api/agent/chat | Agentic AI interaction |
| GET | /api/optimization/{id} | Optimization status/result |

## 17.1 Example optimize request

{

"run_id": "demo-001",

"policy": "CARBON_PRIORITY",

"horizon_steps": 96,

"weights": {

"cost": 0.10,

"carbon": 0.50,

"peak": 0.20,

"discomfort": 0.20

},

"allow_export": false

}

## 17.2 Example optimizer response

{

"solver_status": "OPTIMAL",

"objective_value": 0.1842,

"solve_time_seconds": 0.42,

"peak_grid_import_kw": 1880,

"total_cost": 4210.5,

"total_co2e_kg": 312.7,

"hard_constraint_violations": 0,

"schedule_id": "sched-0042"

}

# 18. End-to-End Operational Workflow

1. CREATE SCENARIO

↓

2. LOAD CONFIGURATION

Community size, assets, battery, PCC limit, policy

↓

3. GENERATE / LOAD BASELINE DATA

Residential load + flexible workloads + solar + grid signals

↓

4. FORECAST

Predict next 24h load and renewable availability

↓

5. BUILD CARBON / PRICE SIGNALS

Time-varying gCO2e/kWh and tariff

↓

6. RUN BASELINE

Produce uncontrolled/reference schedule

↓

7. OPTIMIZE

CP-SAT / MIP solves multi-objective constrained schedule

↓

8. VALIDATE

Hard constraints + energy balance + battery consistency

↓

9. SIMULATE

Execute schedule against actual synthetic conditions

↓

10. EVALUATE

Peak, cost, CO2e, renewable utilization, violations

↓

11. AGENT INTERFACE

Explain, compare, answer what-if questions

↓

12. VISUALIZE

Dashboard + schedule timeline + metric deltas

# 19. Frontend / UX Specification

## 19.1 Main dashboard

| Panel | Required content |
| --- | --- |
| Grid status | PCC import, peak, capacity utilization, grid status |
| Carbon | Current and 24h carbon-intensity chart, current gCO₂e/kWh |
| Renewables | Solar output, renewable share, forecast |
| Battery | SOC %, charge/discharge, reserve |
| Demand | Community load, flexible load, baseline vs optimized |
| Schedule | 96-step timeline by asset/load class |
| Impact | Cost, CO₂e, peak and renewable-utilization deltas |
| AI Copilot | Natural-language agent interaction and explanation |
| Scenario controls | Weather factor, EV count, battery size, policy profile |

## 19.2 Recommended visual hierarchy

TOP ROW:

[Grid Load] [Peak] [Carbon Intensity] [Renewables] [Battery SOC]

MIDDLE:

[24h Demand / Solar / Grid Import chart]

[Carbon Intensity chart]

LOWER:

[Flexible Load Schedule timeline]

[Battery dispatch chart]

[Baseline vs Optimized metrics]

RIGHT / FLOATING:

[AI Energy Copilot]

## 19.3 Required interactions

Toggle Baseline / Optimized.

Select policy: Balanced / Carbon Priority / Peak Protection.

Run What-If: cloudy day, +30 EVs, larger data center, reduced battery capacity, high evening demand.

Click a schedule action to see why it moved.

Ask the AI agent a question in natural language.

Reset scenario to a seeded baseline.

# 20. Demonstration Scenarios

| Scenario | Change | Expected system behavior |
| --- | --- | --- |
| S1 Normal Day | Default configuration | Optimize cost/carbon/peak jointly. |
| S2 Solar-Rich Day | High solar factor | Shift flexible loads into solar-rich periods; charge battery where useful. |
| S3 Cloudy Day | Solar output -40% | Preserve battery, shift flexible loads, increase grid import only as needed. |
| S4 Evening Peak | Residential demand +25% from 18:00–21:00 | Pre-charge battery and shift flexible loads away from peak. |
| S5 EV Surge | EV sessions +30 | Distribute charging across available windows while meeting departure deadlines. |
| S6 Data-Center Surge | Flexible compute +100 kW | Use deadline-aware workload shifting and battery/grid coordination. |
| S7 Carbon Priority | Increase carbon weight | Accept a possible cost trade-off to reduce modeled CO₂e. |
| S8 Peak Protection | Tighten PCC limit | Prevent import from exceeding capacity; delay/shift flexible demand. |

# 21. Evaluation Metrics

## 21.1 Core metrics

Peak demand = max_t(GridImportPower_t)

Total energy imported = Σ_t(GridImportPower_t × Δt)

Energy cost = Σ_t(GridImportEnergy_t × Price_t)

CO2e = Σ_t(GridImportEnergy_t × CarbonIntensity_t)

Renewable utilization =

RenewableEnergyUsedLocally / RenewableEnergyGenerated

Peak reduction (%) =

(BaselinePeak - OptimizedPeak) / BaselinePeak × 100

CO2e reduction (%) =

(BaselineCO2e - OptimizedCO2e) / BaselineCO2e × 100

Cost reduction (%) =

(BaselineCost - OptimizedCost) / BaselineCost × 100

## 21.2 Constraint metrics

Number of hard-constraint violations.

Number of missed deadlines.

EV sessions not fully charged by departure.

Battery SOC bound violations.

PCC import-capacity violations.

Unserved critical load energy.

User comfort deviation for HVAC.

## 21.3 AI/agent metrics

Tool-call correctness: did the agent select the required tool?

State grounding: were system claims backed by tool outputs?

Optimization success rate.

What-if scenario completion rate.

Explanation completeness: schedule change, reason, impact, constraint status.

# 22. Validation and Verification Plan

| Test | Expected result |
| --- | --- |
| Energy balance | Demand = solar direct use + battery discharge + grid import - battery charge - export, within numerical tolerance. |
| Battery SOC | SOC remains inside configured bounds. |
| No simultaneous battery charge/discharge | Never true in a valid optimized schedule. |
| EV deadline | Every feasible EV receives required energy before departure. |
| PCC constraint | Grid import never exceeds PCC limit. |
| Critical load | Critical demand remains served. |
| Baseline reproducibility | Same seed produces identical baseline time series. |
| Scenario isolation | What-if run does not mutate parent scenario. |
| Infeasible case | Optimizer returns INFEASIBLE/appropriate status and agent explains why. |
| Agent grounding | Agent cannot report a value not returned by an approved tool. |

# 23. Technology Stack

| Layer | Technology | Reason |
| --- | --- | --- |
| Frontend | Next.js + TypeScript | Fast interactive dashboard and strong developer ecosystem. |
| UI | Tailwind CSS + shadcn/ui | Rapid professional UI construction. |
| Charts | Recharts | Simple time-series and comparison visualization. |
| Backend | Python + FastAPI | Natural fit for simulation, ML and optimization. |
| Numerical | NumPy + Pandas | Time-series and simulation calculations. |
| Optimization | Google OR-Tools | Scheduling/constraint/MIP capabilities. |
| ML | XGBoost / LightGBM | Fast tabular forecasting for a prototype. |
| LLM | OpenAI API | Natural-language reasoning and tool-calling agent. |
| Storage | SQLite | Zero-ops local development. |
| Config | YAML/JSON | Scenario reproducibility and parameter tuning. |
| Testing | Pytest + API tests | Numerical and workflow verification. |

# 24. Recommended Repository Structure

community-energy-ai/

│

├── backend/

│   ├── app/

│   │   ├── api/

│   │   │   ├── routes_grid.py

│   │   │   ├── routes_simulation.py

│   │   │   ├── routes_optimization.py

│   │   │   └── routes_agent.py

│   │   ├── simulation/

│   │   │   ├── simulator.py

│   │   │   ├── loads.py

│   │   │   ├── solar.py

│   │   │   ├── battery.py

│   │   │   └── grid.py

│   │   ├── forecasting/

│   │   │   ├── features.py

│   │   │   ├── train.py

│   │   │   └── predict.py

│   │   ├── optimization/

│   │   │   ├── model.py

│   │   │   ├── constraints.py

│   │   │   └── solver.py

│   │   ├── carbon/

│   │   │   └── intensity.py

│   │   ├── agent/

│   │   │   ├── agent.py

│   │   │   ├── tools.py

│   │   │   └── prompts.py

│   │   ├── evaluation/

│   │   │   └── metrics.py

│   │   └── main.py

│   ├── configs/

│   │   ├── default.yaml

│   │   ├── scenarios.yaml

│   │   └── policies.yaml

│   └── tests/

│

├── frontend/

│   ├── app/

│   │   ├── page.tsx

│   │   ├── scenarios/

│   │   └── api/

│   ├── components/

│   │   ├── GridStatus.tsx

│   │   ├── EnergyChart.tsx

│   │   ├── CarbonChart.tsx

│   │   ├── BatteryCard.tsx

│   │   ├── ScheduleTimeline.tsx

│   │   ├── ImpactMetrics.tsx

│   │   └── AICopilot.tsx

│   └── lib/

│

├── data/

│   ├── generated/

│   └── models/

│

├── README.md

└── docker-compose.yml  (optional)

# 25. Core Algorithm Pseudocode

def run_energy_orchestration(config, scenario):

state = simulator.initialize(config, scenario)

forecast = forecasting.predict(

historical=state.history,

horizon=96

)

carbon = carbon_model.generate(

horizon=96,

scenario=scenario

)

baseline = baseline_scheduler.build(

state=state,

horizon=96

)

optimized = optimizer.solve(

demand_forecast=forecast.load,

solar_forecast=forecast.solar,

carbon_profile=carbon,

price_profile=state.price,

flexible_loads=state.flexible_loads,

battery=state.battery,

grid_limit=state.pcc_limit,

policy=scenario.policy

)

validator.validate(optimized)

baseline_result = simulator.execute(

schedule=baseline,

actual_state=state

)

optimized_result = simulator.execute(

schedule=optimized,

actual_state=state

)

metrics = evaluator.compare(

baseline_result,

optimized_result

)

return {

"baseline": baseline_result,

"optimized": optimized_result,

"metrics": metrics

}

# 26. What-If / Digital-Twin Scenario Engine

A scenario is a copy of a parent simulation configuration plus explicit changes. The scenario engine must never mutate the parent run. This supports controlled experiments and makes the AI agent's recommendations auditable.

| Scenario parameter | Example values |
| --- | --- |
| solar_multiplier | 0.6, 1.0, 1.2 |
| residential_demand_multiplier | 0.8–1.3 |
| ev_count | 20, 40, 60, 80 |
| data_center_power_kw | 100, 200, 300, 400 |
| battery_capacity_kwh | 250, 500, 1000 |
| pcc_limit_kw | 1800, 2000, 2500 |
| carbon_multiplier | 0.8–1.5 |
| price_multiplier | 0.8–1.5 |
| policy | BALANCED / CARBON / PEAK |

## 26.1 Example What-If request

"What happens if tomorrow's solar generation drops by 40% and 20 more EVs arrive?"

Agent actions:

1. clone baseline scenario

2. solar_multiplier = 0.60

3. ev_count = baseline + 20

4. regenerate flexible-load requirements

5. forecast

6. optimize

7. simulate

8. compare against baseline

9. explain trade-offs

# 27. Industry Alignment and Schneider-Relevant Context

The architecture intentionally resembles the functional separation used in modern digital-grid and energy-management systems: operational state, forecasting, optimization, DER/flexibility management, human decision support, and digital twin/simulation. Schneider Electric publicly describes its One Digital Grid Platform as an AI-enabled platform combining planning, operations and asset management, with capabilities including Grid AI Assistant, AI-based network model tuning, and real-time outage/restoration intelligence.

Schneider also describes its grid ecosystem in terms of ADMS and DERMS, distributed energy resources, grid flexibility, predictive analytics and AI-enabled operations. This proposal should therefore use terms such as DER, flexibility, forecasting, optimization, digital twin, grid operations and decision support accurately rather than presenting the project as a generic chatbot.

A current Schneider Electric Data Scientist posting describes an AI Core team working on the Grid AI Assistant inside ADMS and lists responsibilities including software-development lifecycle participation and collecting, preprocessing and analyzing data for AI models. This is relevant context for positioning the project as an AI/software energy-domain prototype.

Positioning statement The project is not a reproduction of Schneider's proprietary software. It is an independently designed academic/prototype system using publicly described industry concepts to demonstrate forecasting, constrained optimization, DER coordination, digital-twin simulation and agentic AI.

# 28. Reliability, Security and Responsible AI Design

Never expose the OpenAI API key to the frontend.

Validate every tool input with server-side schemas.

Keep physical/operational state read-only to the LLM.

Do not permit the agent to bypass hard constraints.

Return solver status and validation status with every optimized schedule.

Use deterministic seeds for simulation and test runs.

Log agent tool calls for auditability.

Keep scenario data isolated.

Clearly label modeled carbon intensity and synthetic demand.

Treat the system as decision support, not autonomous safety-critical grid control.

# 29. Two-Day Implementation Plan

| Time | Deliverable | Definition of done |
| --- | --- | --- |
| Hour 0–2 | Project skeleton | FastAPI + Next.js + config + database running. |
| Hour 2–5 | Simulation engine | 96-step demand, solar, battery and grid simulation. |
| Hour 5–7 | Flexible loads | EV, data center, pump/HVAC workloads with constraints. |
| Hour 7–10 | Baseline + metrics | Baseline schedule and cost/carbon/peak calculations. |
| Hour 10–14 | Optimizer | OR-Tools schedule with hard constraints and policy weights. |
| Hour 14–16 | Dashboard v1 | Charts, KPIs, baseline/optimized comparison. |
| Hour 16–20 | Forecasting | Synthetic history + lightweight ML forecasts. |
| Hour 20–24 | Agent tools | Grid state, forecast, optimize, simulate, compare tools. |
| Hour 24–28 | LLM integration | Natural-language agent with tool calling. |
| Hour 28–32 | What-if scenarios | Cloudy day, EV surge, peak demand scenarios. |
| Hour 32–36 | UI polish | AI Copilot, timeline, scenario controls, explanation cards. |
| Hour 36–40 | Testing | Constraint validation, reproducibility, infeasible-case test. |
| Hour 40–48 | Demo hardening | Seeded demo, README, screenshots, presentation metrics. |

Time constraint strategy If time runs short, do not remove the optimizer or the simulator. Remove secondary features first: wind, authentication, complex persistence, live external feeds, and sophisticated forecasting. The minimum credible system is simulation + baseline + constrained optimization + metrics + agent tool calling + dashboard.

# 30. AI-Assisted Development Strategy

Because the implementation may be generated heavily with AI coding assistance, the developer should retain ownership of the architecture and verification. Generated code must be accepted only after it passes numerical tests.

| AI can generate | Developer must understand/verify |
| --- | --- |
| Boilerplate FastAPI routes | API contracts and data flow |
| React components | What each metric means |
| Synthetic data generators | Units and physical plausibility |
| OR-Tools model code | Decision variables, constraints and objective |
| Agent tool schemas | Tool boundaries and security |
| Chart code | Whether plotted values match backend results |
| Tests | Whether tests cover real invariants |

# 31. Final Deliverables

Working Next.js dashboard.

Working FastAPI backend.

Seeded 24-hour/96-step simulation.

Baseline scheduler.

Constraint-based optimizer.

Load and solar forecasting module.

Carbon-intensity and emissions accounting module.

Battery dispatch module.

Agentic AI with OpenAI tool calling.

What-if scenario engine.

Baseline vs optimized evaluation report.

Configuration files for community parameters and policy weights.

Automated validation tests.

README with architecture and run instructions.

Short demo script showing at least three scenarios.

# 32. Recommended 5-Minute Review Demonstration

Open the dashboard and show the virtual community: apartments, EVs, data center, solar, battery and grid connection.

Run the baseline scenario and point out the evening peak, battery state and carbon-intensity curve.

Run Balanced optimization and show that deadlines remain satisfied while peak/cost/carbon change.

Switch to Carbon Priority and show how the objective weights change scheduling behavior.

Ask the AI agent: “Why did you move these EV charging sessions?”

Ask: “What if tomorrow is 40% cloudier?” The agent should run a new scenario and explain the impact.

Ask: “What if 20 more EVs join?” Show the new peak and the optimizer's response.

Show the validation panel: solver status, hard-constraint violations = 0, energy-balance check = PASS.

Conclude with baseline vs optimized metrics and explain that the physical grid is simulated while the decision architecture is industry-aligned.

# 33. Risks and Mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Overly complex physical model | Schedule slips | Use energy-balance abstraction, not full power flow. |
| LLM makes numerical mistakes | Incorrect decisions | LLM cannot be the optimizer; use tools + solver. |
| Synthetic data looks unrealistic | Weak review | Use clear assumptions, smooth profiles, seeded noise and sanity checks. |
| Optimizer infeasible | Demo failure | Include fallback baseline and infeasibility explanation. |
| Forecasting takes too long | Reduced polish | Use simple gradient boosting or persistence baseline. |
| UI consumes too much time | Incomplete backend | Build dashboard after core simulation/optimizer works. |
| API failure | Agent unavailable | System remains fully usable without agent; optimization is independent. |

# 34. Future Extension Roadmap

Live weather and grid carbon-intensity feeds.

Real smart-meter data ingestion.

DERMS-style aggregation of batteries, EVs and flexible loads.

Real-time streaming telemetry.

Physics-based power-flow simulation using OpenDSS, pandapower or ETAP-class tooling.

Forecast uncertainty and robust/stochastic optimization.

Reinforcement learning for policy optimization after a deterministic baseline is established.

Federated/privacy-preserving apartment-level learning.

Building Management System / EV charging API integration.

Hardware-in-the-loop simulation.

Cloud deployment with observability and role-based access.

# 35. Glossary

| Term | Meaning in this project |
| --- | --- |
| DER | Distributed Energy Resource such as rooftop solar, battery or EV. |
| EMS | Energy Management System; coordinates energy resources and loads. |
| ADMS | Advanced Distribution Management System; utility distribution operations software. |
| DERMS | Distributed Energy Resource Management System. |
| PCC | Point of Common Coupling / community grid connection. |
| Peak shaving | Reducing maximum grid/community demand during high-load periods. |
| Carbon intensity | Modeled emissions associated with one unit of grid electricity, e.g. gCO₂e/kWh. |
| Demand response | Changing flexible electricity consumption in response to grid/economic signals. |
| Digital twin | A software representation of an operational system used for monitoring, analysis or simulation. |
| CP-SAT | Constraint programming solver in OR-Tools suitable for discrete scheduling/optimization. |
| Agentic AI | An AI system that can interpret goals and invoke tools/actions in a controlled workflow. |
| LLM | Large Language Model used here for language understanding, tool selection and explanation. |
| Flexible load | Electrical demand whose timing can be changed without violating constraints. |
| SOC | Battery state of charge. |
| CO₂e | Carbon-dioxide equivalent used to aggregate greenhouse-gas impacts into a common unit. |

# 36. Final Acceptance Criteria

The simulator produces a reproducible 96-step virtual community day.

The baseline produces a valid demand/grid/battery trajectory.

The optimizer returns a feasible schedule for the default scenario.

No hard constraints are violated by the optimized schedule.

Baseline and optimized runs use the same underlying scenario realization.

Peak, cost and carbon metrics are computed from time-series results rather than hard-coded values.

At least one scenario demonstrates peak shaving.

At least one scenario demonstrates carbon-aware scheduling.

At least one scenario demonstrates data-center workload flexibility.

At least one scenario demonstrates battery dispatch.

The agent can inspect state and call optimization tools.

The agent can answer a why/what-if question using actual tool outputs.

The dashboard visibly distinguishes baseline and optimized operation.

All synthetic assumptions are configurable.

The README explains how to reproduce the demo.

# 37. Reference Basis

The project design uses the following authoritative or primary references for terminology and architectural context. The references support the distinction between grid operations, AI-enabled digital-grid software, DER/flexibility management, and mathematical scheduling/optimization.

Schneider Electric — One Digital Grid Platform
AI-enabled grid platform integrating planning, operations and asset management; describes Grid AI Assistant, AI-based network model tuning, ADMS/DERMS and real-time data use.
https://www.se.com/ww/en/about-us/newsroom/news/press-releases/schneider-electric-debuts-one-digital-grid-platform-to-help-utilities-modernize-and-address-energy-costs-691af6851937b58c890951a3/

Schneider Electric — Future of Energy Intelligence
Overview of grid planning, operations/resiliency and grid flexibility/prosumer engagement.
https://www.se.com/ww/en/work/campaign/energy-intelligence/

Schneider Electric — Data Scientist / AI Core
Current Schneider careers description of an AI Core team working on Grid AI Assistant inside ADMS.
https://careers.se.com/jobs/130856?lang=en-us

Google OR-Tools
Official optimization toolkit documentation covering scheduling, constraint optimization and mathematical programming.
https://developers.google.com/optimization

Google OR-Tools — CP-SAT
Official CP-SAT solver documentation and solver-status behavior.
https://developers.google.com/optimization/cp/cp_solver

# Appendix A — Default Configuration

simulation:

timestep_minutes: 15

horizon_steps: 96

random_seed: 42

community:

apartments: 100

ev_chargers: 40

solar_capacity_kw: 250

battery_capacity_kwh: 500

battery_power_kw: 250

pcc_limit_kw: 2500

battery:

min_soc_pct: 20

max_soc_pct: 95

charge_efficiency: 0.95

discharge_efficiency: 0.95

flexible_loads:

ev:

charger_kw: 7.2

min_energy_kwh: 8

max_energy_kwh: 24

data_center:

flexible_power_kw: 200

default_duration_hours: 2

water_pump:

power_kw: 50

required_energy_kwh: 50

policy:

balanced:

cost: 0.25

carbon: 0.25

peak: 0.25

discomfort: 0.20

curtailment: 0.05

carbon:

baseline_g_per_kwh: 450

solar_day_reduction_factor: 0.55

evening_peak_multiplier: 1.35

# Appendix B — Example Decision Logic

IF flexible load has a deadline:

schedule enough energy before deadline

IF PCC demand approaches limit:

shift low-priority flexible loads

discharge battery if economically/operationally justified

IF solar forecast is high:

prioritize local flexible consumption

charge battery if future peak value exceeds immediate use

IF carbon intensity is low:

prefer flexible consumption if it does not create a new peak

IF battery SOC approaches reserve:

prohibit further discharge

IF optimization is infeasible:

return infeasible status

preserve critical loads

fall back to validated baseline / emergency policy

explain the limiting constraint

# Appendix C — One-Sentence Project Definition

Project definition A software digital twin of a renewable-enabled community that uses ML forecasting, deterministic constrained optimization, battery/DER coordination, carbon-aware scheduling and an Agentic AI interface to reduce grid peaks and modeled operational emissions while satisfying residential, commercial and data-center workload constraints.